# 96-well Miniprep Protocol

## Materials & Buffers

| Reagent / Buffer | Volume per Well | Notes |
| --- | --- | --- |
| TB + Antibiotic | 1.2 mL culture | Pre-warmed to 37 °C |
| P1 (Resuspension) | 250 µL | Qiagen buffer + RNase A |
| P2 (Lysis) | 250 µL | Alkaline lysis |
| N3 (Neutralization) | 350 µL | Silica-binding enhancer |
| PE (Wash) | 750 µL × 2 | Ethanol wash buffer |
| ddH₂O (Elution) | 70 µL heated to 60 °C | Pre-heat to promote yield |

## Equipment

- 2 mL square well deep-well 96-well plate (for growth & wash steps)

- 96-well lysate clearing plate (Epoch 2021F-002)

- 96-well silica binding plate (Epoch 2020-001)

- Swinging-bucket centrifuge (up to 5,000×g) with adapter for 96-well plates

- Multichannel pipettes & tip

- 60C water bath

- 96-well PCR plate

## Workflow

### 1. Culture & Harvest

- Grow cultures in 1.2 mL TB + antibiotic in each well (2 mL deep-well plate) overnight at 37 °C and 250 RPM.

- These cultures are inoculated from the Addgene glycerol stocks.

- Pellet cells: 5,000×g for 10 min (swinging-bucket).

- Discard the supernatant without disturbing the pellet (flipping the plate should work if your spin speed was sufficient).

### 2. Lysis & Neutralization (use NEB miniprep buffers)

- Resuspend pellet in 200 µL B1, pipette up/down until no clumps remain.

- Lyse: add 200 µL B2, mix 6–8× by pipetting, incubate 2 min at room temp.

- Neutralize: add 400 µL B3, mix 6–8×, until solution looks uniform.

### 3. Lysate Clarification & Binding

- Clarify

  - Spin at 5,000×g for 30 min (deep-well plate).

  - Transfer only the clear lysate to a 96-well lysate-clearing plate, and spin at 5,000×g for 5 minutes.

- Bind DNA

  - Place binding plate on vacuum manifold (or spin adaptor).

  - Flow lysate through the binding plate—spin at 5,000xg for 5 minutes and collect flow-through in a waste plate.

### 4. Wash & Dry

- Wash × 2

  - Wash with 200 µL Buffer BZ

  - Wash with 400 µL Buffer WZ

- Dry spin

  - Centrifuge for 5 min to remove residual ethanol.

### 5. Elution & QC

- Elute

  - Warm ddH₂O to 60 °C, then add 70 µL to each well, wait 5 min.

  - Spin at 5,000×g for 5 min into a fresh 96-well PCR plate.

- Quantify DNA (e.g., UV/fluorescence).

  - You can use the SpectraMax Quant AccuClear Nano dsDNA Assay Kit. Find a protocol here.

- Store at -20 °C until ready for downstream assemblies.

## Tips for Success

- Pipette consistency: always use a multichannel and keep tips at a uniform depth.

- Balance troubleshooting: when spinning empty wells, add dummy weights (e.g., unused tips) to keep the rotor balanced.

- Prevent clogging: if lysate clogs a column, brief spin-down of the binding plate will clear debris—just repeat vacuum.

- Scale & iterate: once this runs smoothly, you can automate timing (e.g., robotics-friendly pauses) or swap in alternative lysis chemistries for higher-yield strains.